# Five W’s:

What? What are you building — one sentence, no jargon.

A software prototype that instantly saves a sensor's data to permanent memory the millisecond it detects an impending power loss.

Who? Who uses it, and who else is affected?

Hardware engineers designing battery-less medical implants, and eventually the patients who will safely swallow or wear them.

Why? What breaks if it doesn’t exist?

Devices running on weak ambient energy will constantly reboot and erase their progress before they can finish a single biological measurement.

When? When is it used — and when is it needed by?

The survival circuit triggers in the nanoseconds before a power crash; the physical prototype must be ready for the final "pull-the-plug" test by week 12 of the semester.

Where? In what setting or workflow does it live?

On an FPGA development board wired to a custom energy-harvesting circuit that simulates the extreme power limits of the human body.

# Mission Statement

For hardware engineers who need to build tiny, battery-less medical sensors, the Intermittent Computing Prototype is a custom software simulation that guarantees a device can finish its work on weak, flickering power. Unlike traditional computer chips that wipe their memory and reboot every time the voltage drops, it treats power loss as a simple pause—instantly saving the sensor's exact progress to permanent memory and picking up right where it left off when energy returns.

# User Stories and Chart

### 1 & 2. Assumptions (Ranked: Most Fatal × Most Uncertain First)

- The Detection Window (Most Fatal × Most Uncertain): We can consistently detect a collapsing voltage rail and trigger a hardware interrupt fast enough to give the FPGA time to act before the voltage drops below the SRAM physical retention limit.

- The Write Deadline (Fatal × Moderately Uncertain): We can complete a full SPI serialization and write transaction to the external FRAM within the tiny microsecond energy window provided by the dying capacitor.

- The Simulation Constraint (Uncertain × High Impact): We can accurately simulate a stochastic, weak ambient energy harvester (a fluctuating current source) on a breadboard to predictably test the FPGA's response.

- The Value Proposition (Problem/Need): Hardware engineers designing biological edge devices lack a reliable RTL-level template for intermittent state retention, currently relying on slow, power-hungry software interrupts that fail under extreme constraints.

### 3. The Cheapest Test (For the #1 Assumption)

Do not write SystemVerilog yet.

Build the physical energy plane on a breadboard: wire a capacitor, a resistor (to simulate the FPGA's power draw), and an analog voltage comparator. Purposely cut the power. Use an oscilloscope to measure the exact time delta ($\Delta t$) between the comparator triggering the "brownout" interrupt and the capacitor voltage crossing the FPGA's minimum SRAM retention threshold.

Proof: If $\Delta t$ is shorter than the physical time it takes to clock our data over an SPI bus, the architecture is physically impossible to build.

### 4. The Kill Criterion

We pivot (or drastically redesign the hardware power plane) if:

The oscilloscope measurement shows the voltage collapses from the interrupt threshold to the SRAM failure threshold in less time than it takes to transmit 32 bits of state data over SPI at our maximum reliable clock frequency.

### 5. GitHub Repo Deliverables for Next Class

Mission Sentence

For hardware engineers who need to build tiny, battery-less medical sensors, the Intermittent Computing Prototype is a custom hardware architecture that guarantees a device can finish its work on weak, flickering power. Unlike traditional computer chips that wipe their memory and reboot every time the voltage drops, it treats power loss as a simple pause—instantly saving the sensor's exact progress to permanent memory and picking up right where it left off when energy returns.

Named User

Hardware Systems Architect for in vivo biological edge devices.

Top-5 User Stories with Acceptance Criteria

- Story 1: The Hardware Interrupt

  - As an RTL designer, I want a dedicated "Survival Controller" FSM that listens for a low-voltage interrupt, so that the chip can instantly halt the main processor.

  - Acceptance Criteria:

    - Interrupt pin going HIGH transitions the FSM from ACTIVE to HALT state.

    - Clock gating successfully pauses the main datapath within 2 clock cycles.

- Story 2: The Emergency Save

  - As a systems engineer, I want the FPGA to automatically serialize and blast all in-progress data to the FRAM, so that the chip doesn't lose its intermediate math.

  - Acceptance Criteria:

    - FSM transitions to FLUSH_NVM.

    - SPI controller successfully writes the entire 32-bit state register block to the external FRAM.

    - Write completes before the system voltage drops below the operational threshold.

- Story 3: The Resumption Boot Sequence

  - As an RTL designer, I want the chip's boot-up sequence to default to checking the FRAM first, so that the system restores its saved progress rather than starting from zero.

  - Acceptance Criteria:

    - Power-On Reset (POR) defaults to a RESTORE state, not an IDLE/RESET state.

    - SPI controller reads the previous state from FRAM and populates the active registers before enabling the main datapath clock.

- Story 4: The Harvester Simulation

  - As a hardware engineer, I want to build a specialized low-voltage RC circuit, so that I can accurately simulate the trickle-charge and rapid depletion of a micro-harvester.

  - Acceptance Criteria:

    - Circuit charges the main capacitor at < 3V.

    - A manual toggle switch successfully cuts input power, simulating a "passing shadow" over a solar cell or a temperature drop.

- Story 5: The Pull-the-Plug Torture Test

  - As a test engineer, I want to introduce artificial power cuts to the FPGA while it is running a simulated workload, so that I can physically prove the device successfully saves, dies, and resumes.

  - Acceptance Criteria:

    - FPGA runs a continuous counter/math operation.

    - Power is manually cut and restored 25 times at random intervals.

    - The final mathematical output perfectly matches a software-simulated baseline, proving zero data loss across 25 power deaths.

Assumption Table & Test Results

| Assumption | Risk Rank | Test Method | Kill Criterion | Status/Result |
| --- | --- | --- | --- | --- |
| Voltage Detection Window | 1 (Fatal/High) | Oscilloscope measurement of RC discharge curve vs. SPI clock speed. | $\Delta t$ is shorter than a 32-bit SPI transaction. | Pending test execution |
| Write Deadline | 2 (Fatal/Med) | SystemVerilog simulation of FSM clock cycles required to complete the SPI write. | SPI state machine requires more clock cycles than the power window allows. | Pending test execution |
| Harvester Simulation | 3 (Med/Med) | Build breadboard power plane; measure charge/discharge with multimeter. | Cannot create a predictable brownout curve. | Pending test execution |